# Number System and Base Conversions

Electronic and digital systems use various number systems such as Decimal, Binary, Hexadecimal and Octal, which are essential in computing.

- Binary (base-2) is the foundation of digital systems.

- Hexadecimal (base-16) and Octal (base-8) are commonly used to simplify the representation of binary data.

- The Decimal system (base-10) is the standard system for everyday calculations.

- Other number systems like Duodecimal (base-12), are less commonly used but have specific applications in certain fields.

## What is Number System ?

- A number system is a way to represent and express numbers using a consistent set of symbols or digits.

- A number system uses a base (or radix) to represent values.

- The base refers to the number of unique digits, including zero, that a system uses to represent numbers.

- Most commonly used number systems are Decimal (base-10), Binary (base-2), Octal (base-8), and Hexadecimal (base-16).

- Each system has its own set of rules for representing.

- Digital systems primarily rely on the Binary system for data processing.

## Types of Number System

There are four common types of number systems based on the radix or base of the number :

### 1. Decimal Number System

- The Decimal system is a base-10 number system.

- It uses ten digits: 0, 1, 2, 3, 4, 5, 6, 7, 8 and 9.

- Each digit’s place value is a power of 10 (e.g., 100, 101, 102).

- It is the standard system for everyday counting and calculations.

### 2. Binary Number System

- The Binary system is a base-2 number system.

- It uses two digits: 0 and 1.

- Each digit’s place value is a power of 2 (e.g., 20, 21, 22).

- The Binary system is the foundation for data representation in computers and digital electronics.

### 3. Octal Number System

- The Octal system is a base-8 number system.

- It uses eight digits: 0, 1, 2, 3, 4, 5, 6 and 7.

- Each digit’s place value is a power of 8 (e.g., 80, 81, 82).

- It is often used to simplify the representation of binary numbers by grouping them into sets of three bits.

### 4. Hexadecimal Number System

- The Hexadecimal system is a base-16 number system.

- It uses sixteen digits: 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, A, B, C, D, E and F (where A = 10, B = 11, etc.).

- Each digit’s place value is a power of 16 (e.g., 160, 161, 162).

- Hexadecimal simplifies binary by representing every 4 bits as one digit (0-F).

## Number System Conversion Methods

A number N in base or radix b can be written as:

(N)b = dn-1 dn-2 -- -- -- -- d1 d0 . d-1 d-2 -- -- -- -- d-m

In the above, dn-1 to d0 is the integer part, then follows a radix point and then d-1 to d-m is the fractional part. 

dn-1 = Most significant bit (MSB) 
d-m = Least significant bit (LSB)

### 1. Decimal to Binary Number System Conversion

For Integer Part:

- Divide the decimal number by 2.

- Record the remainder (0 or 1).

- Continue dividing the quotient by 2 until the quotient is 0.

- The binary equivalent is the remainders read from bottom to top.

For Fractional Part:

- Multiply the fractional part by 2.

- Record the integer part (0 or 1).

- Take the fractional part of the result and repeat the multiplication.

- Continue until the fractional part becomes 0 or reaches the desired precision.

- The binary equivalent is the integer parts recorded in sequence.

Example: (10.25)10

For Integer Part (10):

- Divide 10 by 2 → Quotient = 5, Remainder = 0

- Divide 5 by 2 → Quotient = 2, Remainder = 1

- Divide 2 by 2 → Quotient = 1, Remainder = 0

- Divide 1 by 2 → Quotient = 0, Remainder = 1

Reading the remainders from bottom to top gives 1010.

For Fractional Part (0.25):

- Multiply 0.25 by 2 → Result = 0.5, Integer part = 0

- Multiply 0.5 by 2 → Result = 1.0, Integer part = 1

The fractional part ends here as the result is now 0. Reading from top to bottom gives 01.

Thus, the binary equivalent of (10.25)10 is (1010.01)2.

### 2. Binary to Decimal Number System Conversion

For Integer Part:

- Write down the binary number.

- Multiply each digit by 2 raised to the power of its position, starting from 0 (rightmost digit).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the binary integer.

For Fractional Part:

- Write down the binary fraction.

- Multiply each digit by 2 raised to the negative power of its position, starting from -1 (first digit after the decimal point).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the binary fraction.

Example: (1010.01)2

1x23 + 0x22 + 1x21+ 0x20 + 0x2 -1 + 1x2 -2 = 8+0+2+0+0+0.25 = 10.25

Thus, (1010.01)2 = (10.25)10

### 3. Decimal to Octal Number System Conversion

For Integer Part:

- Divide the decimal number by 8.

- Record the remainder (0 to 7).

- Continue dividing the quotient by 8 until the quotient is 0.

- The octal equivalent is the remainders read from bottom to top.

### 4. Octal to Decimal Number System Conversion

For Integer Part:

- Write down the octal number.

- Multiply each digit by 8 raised to the power of its position, starting from 0 (rightmost digit).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the octal integer.

### 5. Decimal to Hexadecimal Conversion

For Integer Part:

- Divide the decimal number by 16.

- Record the remainder (0-9 or A-F).

- Continue dividing the quotient by 16 until the quotient is 0.

- The hexadecimal equivalent is the remainders read from bottom to top.

### 6. Hexadecimal to Decimal Conversion

For Integer Part:

- Write down the hexadecimal number.

- Multiply each digit by 16 raised to the power of its position, starting from 0 (rightmost digit).

- Add up the results of these multiplications.

- The sum is the decimal equivalent of the hexadecimal integer.

### 7. Hexadecimal to Binary Number System Conversion

To convert from Hexadecimal to Binary:

- Each hexadecimal digit (0-9 and A-F) is represented by a 4-bit binary number.

- For each digit in the hexadecimal number, find its corresponding 4-bit binary equivalent and write them down sequentially.

Example: (3A)16

- (3)16 = (0011)2

- (A)16 = (1010)2

Thus, (3A)16 = (00111010)2

### 8. Binary to Hexadecimal Number System Conversion

To convert from Binary to Hexadecimal:

- Start from the rightmost bit and divide the binary number into groups of 4 bits each.

- If the number of bits isn't a multiple of 4, pad the leftmost group with leading zeros.

- Each 4-bit binary group corresponds to a single hexadecimal digit.

- Replace each 4-bit binary group with the corresponding hexadecimal digit.

Example: (1111011011)2

0011 1101 1011
   |      |       |
 3     D     B

Thus, (001111011011 )2 = (3DB)16

### 9. Binary to Octal Number System

To convert from binary to octal:

- Starting from the rightmost bit, divide the binary number into groups of 3 bits.

- If the number of bits is not a multiple of 3, add leading zeros to the leftmost group.

- Each 3-bit binary group corresponds to a single octal digit.

### 10. Octal to Binary Number System Conversion

To convert from octal to binary:

- Each octal digit (0-7) corresponds to a 3-bit binary number.

- For each octal digit, replace it with its corresponding 3-bit binary equivalent.

Example: (153)8

- Break the octal number into digits: 1, 5, 3

- Convert each digit to binary:

  - 1 in octal = 001 in binary

  - 5 in octal = 101 in binary

  - 3 in octal = 011 in binary

Thus, (153)8 = (001101011)2